## PRACTICAL ASSESSMENT 3 (2 HOURS)

### Elements Covered:

- Test computer component functionality

## Instructions:

- Answer ALL questions.

- Attach clear screenshots as evidence.

- Save your report using your Registration Number as the filename.

- Print and submit your final report.

# PRACTICAL TASK

## TASK: CPU STRESS TEST USING Prime95

You are required to perform a CPU stress test to evaluate processor stability and cooling performance.

### Requirements:

- Install and launch Prime95.

- Select either:

  - Blend Test, OR

  - Small FFTs test option.

- Run the stress test for at least 15 minutes.

- Monitor CPU temperatures using a hardware monitoring tool (e.g., HWMonitor or Core Temp).

- Record the following details:

  - CPU Model

  - Number of Cores/Threads

  - Idle Temperature

  - Maximum Temperature reached during test

  - Any system instability (freezing, crashes, errors)

- Capture clear screenshots showing:

  - Prime95 running the stress test

  - CPU temperature readings

- Interpret the results and state:

  - Whether the CPU is stable

  - Whether the cooling system is adequate

  - Any recommendations if temperatures are high

The assessor will proceed to ask you oral questions.